Data broken during FTP test

2006/04/18

- Environment

Server

Machine : X86

OS  : RHEL5GA(VM)

Client

Machine: X86

OS  : RHEL5GA

(Native)

domain_1

send data

xenbr0

data1

data2

Physical NIC

(BCM5715)

data3

domain_2

connect

FTP data is not broken.

FTP data is broken.

Virtual NIC

- Guess

We are guessing this problem as follows.

Frame information and data are collected by the Scatter/Gather off-road function on the NIC

side and the frame is made.

Server(VM)

domain_1                            Dom0                                  NIC

Frame information

data

sk_buff

buff

DMA buff

header

header

Wt Data

Wt Data

The buffer data of Dom0 is liberated with ack from NIC.

Therefore, the buffer is used other forwarding.

domain_1                           Dom0                                  NIC

ack

sk_buff

buff

DMA buff

header

header

Wt Data

Wt Data

Dom0 liberates buff by the response of ack.

The domain_1 maintains the buffer in preparation for the request of

sending again of TCP.

The request of sending again is notified to guest OS when there is a request of

the TCP sending again.

However, it seems to read data from the buffer because data is maintained in the buffer of Dom0 as for NIC and to send it again.

domain_1                            Dom0                                  NIC

sk_buff

buff

DMA buff

Frame 情報

header

Garbage

Wt Data

Garbage

NIC sends data again by the assumption that data maintains in the buffer.

When the SG(Scatter/Gather) flag is turning off, the request of the TCP sending again is notified and even guest OS is notified.

Therefore, FTP data is not broken because it is sent again by guest OS(domain_1).